Computer Networks

Lab Task #10

Saad Ahmad

20P-0051

Task#1

Task #2

| IPv4 Address/Prefix | Network Address | Broadcast Address | Total Number of Host Bits | Total Number of Hosts |
| --- | --- | --- | --- | --- |
| 192.168.100.25/28 | 192.168.100.16 | 192.168.100.31 | 4 | 14 |
| 172.30.10.130/30 | 172.30.10.128 | 172.30.10.131 | 2 | 2 |
| 10.1.113.75/19 | 10.1.96.0 | 10.1.127.255 | 13 | 8190 |
| 198.133.219.250/24 | 198.133.219.0 | 198.133.219.255 | 8 | 254 |

Task #3

Step 1:	Determine the number of subnets in Network Topology A.

- How many subnets are there?       2

- How many bits should you borrow to create the required number of subnets?     1

- How many usable host addresses per subnet are in this addressing scheme?    126

- What is the new subnet mask in dotted decimal format?          255.255.255.128

- How many subnets are available for future use?      _____0______

Step 2:	Record the subnet information.

Fill in the following table with the subnet information:

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.10.0 | 192.168.10.1 | 192.168.10.126 | 192.168.10.127 |
| 1 | 192.168.10.128 | 192.168.10.129 | 192.168.10.254 | 192.168.10.255 |
| 2 |  |  |  |  |

Task #4

Step 1:	Determine the number of subnets in Network Topology B.

- How many subnets are there?       6

- How many bits should you borrow to create the required number of subnets?       3

- How many usable host addresses per subnet are in this addressing scheme?     30

- What is the new subnet mask in dotted decimal format?       255.255.255.224

- How many subnets are available for future use?       2

Step 2:	Record the subnet information.

Fill in the following table with the subnet information:

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.10.0 | 192.168.10.1 | 192.168.10.30 | 192.168.10.31 |
| 1 | 192.168.10.32 | 192.168.10.33 | 192.168.10.62 | 192.168.10.63 |
| 2 | 192.168.10.64 | 192.168.10.65 | 192.168.10.94 | 192.168.10.95 |
| 3 | 192.168.10.96 | 192.168.10.97 | 192.168.10.126 | 192.168.10.127 |
| 4 | 192.168.10.128 | 192.168.10.129 | 192.168.10.158 | 192.168.10.159 |
| 5 | 192.168.10.160 | 192.168.10.161 | 192.168.10.190 | 192.168.10.191 |
| 6 | 192.168.10.192 | 192.168.10.193 | 192.168.10.222 | 192.168.10.223 |
| 7 | 192.168.10.224 | 192.168.10.225 | 192.168.10.254 | 192.168.10.255 |
| 8 |  |  |  |  |

Step 3:	Assign addresses to network devices in the subnets.

a. Fill in the following table with IP addresses and subnet masks for the router interfaces:

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | GigabitEthernet 0/1 | 192.168.10.1 | 255.255.255.224 |
|  | Serial 0/0/0 | 192.168.10.33 | 255.255.255.224 |
|  | Serial 0/0/1 | 192.168.10.65 | 255.255.255.224 |
| R2 | GigabitEthernet 0/1 | 192.168.10.97 | 255.255.255.224 |
|  | Serial 0/0/0 | 192.168.10.34 | 255.255.255.224 |
|  | Serial 0/0/1 | 192.168.10.129 | 255.255.255.224 |
| R3 | GigabitEthernet 0/1 | 192.168.10.161 | 255.255.255.224 |
|  | Serial 0/0/0 | 192.168.10.66 | 255.255.255.224 |
|  | Serial 0/0/1 | 192.168.10.130 | 255.255.255.224 |