Computer Networks

Lab Task #11

Saad Ahmad

20P-0051

Topology A:

Questions:

- Based on the topology, how many subnets are needed?

5

- How many bits must be borrowed to support the number of subnets in the topology?

3 bits

- How many subnets does this create?

8 subnets

- How many usable hosts does this create per subnet?

30 hosts.

- Calculate the binary value for the first five subnets. The first two subnets have been done for you.

| Subnet | Network Address | Bit 7 | Bit 6 | Bit 5 | Bit 4 | Bit 3 | Bit 2 | Bit 1 | Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 192.168.100. | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 192.168.100. | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 192.168.100. | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 192.168.100. | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 4 | 192.168.100. | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

- Calculate the binary and decimal value of the new subnet mask.

| First Octet | Second Octet | Third Octet | Mask Bit 7 | Mask Bit 6 | Mask Bit 5 | Mask Bit 4 | Mask Bit 3 | Mask Bit 2 | Mask Bit 1 | Mask Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11111111 | 11111111 | 11111111 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

| First Octet | Second Octet | Third Octet | Mask Bit 7 | Mask Bit 6 | Mask Bit 5 | Mask Bit 4 | Mask Bit 3 | Mask Bit 2 | Mask Bit 1 | Mask Bit 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Decimal Octet | Second Decimal Octet | Third Decimal Octet | Fourth Decimal Octet |  |  |  |  |  |  |  |
| 255. | 255. | 255. | 224 |  |  |  |  |  |  |  |

- Fill in the Subnet Table, listing the decimal value of all available subnets, the first and last usable host address, and the broadcast address. Repeat until all addresses are listed.

| Subnet Number | Subnet Address | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| 0 | 192.168.100.0 | 192.168.100.1 | 192.168.100.30 | 192.168.100.31 |
| 1 | 192.168.100.32 | 192.168.100.33 | 192.168.100.62 | 192.168.100.63 |
| 2 | 192.168.100.64 | 192.168.100.65 | 192.168.100.94 | 192.168.100.95 |
| 3 | 192.168.100.96 | 192.168.100.97 | 192.168.100.126 | 192.168.100.127 |
| 4 | 192.168.100.128 | 192.168.100.129 | 192.168.100.158 | 192.168.100.159 |
| 5 | 192.168.100.160 | 192.168.100.161 | 192.168.100.190 | 192.168.100.191 |
| 6 | 192.168.100.192 | 192.168.100.193 | 192.168.100.222 | 192.168.100.223 |
| 7 | 192.168.100.224 | 192.168.100.225 | 192.168.100.254 | 192.168.100.255 |
| 8 |  |  |  |  |

Step 2: Assign the subnets to the network shown in the topology.

- Assign Subnet 0 to the LAN connected to the GigabitEthernet 0/0 interface of R1:

192.168.100.0 /27

- Assign Subnet 1 to the LAN connected to the GigabitEthernet 0/1 interface of R1:

192.168.100.32 /27

- Assign Subnet 2 to the LAN connected to the GigabitEthernet 0/0 interface of R2:

192.168.100.64 /27

- Assign Subnet 3 to the LAN connected to the GigabitEthernet 0/1 interface of R2:

192.168.100.96 /27

- Assign Subnet 4 to the WAN link between R1 to R2:

192.168.100.128 /27

Step 3: Document the addressing scheme.

Fill in the Addressing Table using the following guidelines:

| Device e | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.100.1 | 255.255.255.224 | Nile |
|  | G0/1 | 192.168.100.33 | 255.255.255.224 | Nile |
|  | S0/0/0 | 192.168.100.129 | 255.255.255.224 | Nile |
| R2 | G0/0 | 192.168.100.65 | 255.255.255.224 | Nile |
|  | G0/1 | 192.168.100.97 | 255.255.255.224 | Nile |
|  | S0/0/0 | 192.168.100.130 | 255.255.255.224 | Nile |
| PC0 | Nile | 192.168.100.2 | 255.255.255.224 | 192.168.100.1 |
| PC1 | Nile | 192.168.100.34 | 255.255.255.224 | 192.168.100.33 |
| PC2 | Nile | 192.168.100.66 | 255.255.255.224 | 192.168.100.65 |
| PC3 | Nile | 192.168.100.98 | 255.255.255.224 | 192.168.100.97 |

Part 2: Implement given topology in Packet Tracer and Assign IP Addresses to Network Devices and Verify Connectivity.

Add 4x PCs, 4x Switches and 2x Routers and connect them

Now we will assign the IPs to the PCs

And now we will configure the routes

Router# 1

Router# 2

And now we will configure the static IPs

Router# 1

Router# 2

Verifying the connection between the PCs

Form PC0

From PC2

Topology B:

Add 6x PCs, 3x Switches, 3x Routers and connect them.

Now we will assign the IPs to the PCs

And now we will configure the Routers

Router#1

Router#2

Router#3

And now we will do the static routing on each router

Router#1

Router#2

Router#3

Checking the connection between the PCs on different networks

Form PC#B

From PC#D

From PC#F